UNIT 3 MINI PROJECT REPORT

Introduction to Networking and Data Communication

Project Title:

Configure a DNS Server and Web Server Using Cisco Packet Tracer

Name: Yashraj Pramod Naik

PRN No.: 2125UCSM1055

Class: SY Cyber Security

Subject: Network Protocol Configuration

1. Aim

To configure a DNS server and a web server in Cisco Packet Tracer and access a website using its domain name.

2. Requirements

- Cisco Packet Tracer

- Two PCs

- One Cisco 2960 switch

- One DNS server

- One web server

- Copper straight-through cables

3. IP Address Configuration

| Device | IP Address | Subnet Mask |
| --- | --- | --- |
| DNS Server | 192.168.1.2 | 255.255.255.0 |
| Web Server | 192.168.1.3 | 255.255.255.0 |
| PC0 | 192.168.1.10 | 255.255.255.0 |
| PC1 | 192.168.1.11 | 255.255.255.0 |

4. Procedure

- Created a network topology using two PCs, a switch, a DNS server, and a web server.

- Connected all devices to the switch using copper straight-through cables.

- Assigned static IP addresses to all devices.

- Enabled HTTP service on the web server and created a welcome webpage.

- Enabled DNS service and added the A record www.npc.com mapped to 192.168.1.3.

- Configured both PCs to use DNS server 192.168.1.2.

- Tested network connectivity using the ping command.

- Opened http://www.npc.com in the PC web browser.

5. Screenshots

Topology

PC0 configuration

PC1 configuration

Web server configuratin

DNS server configuration

Ping result

Website opened using its domain name.

6. Result

The DNS server and web server were configured successfully. The ping test returned successful replies, and the website opened using the domain name www.npc.com.

7. Conclusion

This assignment demonstrated how DNS resolves a domain name to an IP address and how a web server delivers a webpage using HTTP. The successful connectivity and browser tests verified the configuration.